PROM file images:  TDS402DV.INT  TDS2020F ANS Forth version 4.02DV in Intel hex format  TDS402DV.MOT  TDS2020F ANS Forth version 4.021DV in Motorola S-record format  TDS402.INT    Alternative Forth ROM      4.02   in Intel hex format  TDS402.MOT    Alternative Forth ROM      4.02   in Motorola S-record formatThe files listed are images in PROM programmer format of TDS2020F Flash ForthROMs. You can make a new Forth ROM by downloading the appropriate file to aPROM programmer (for example using the F12 key in TDS-PC for Windows).Each file represents 4000 (hex) bytes of microprocessor code. Place it asfollows:In a 27C128 EPROM:          whole device     from address (hex) 0000 to 3FFF,  use it in a TDS2020DV piggy-back board on the TDS2020F card.In a 27C256 EPROM:          top    of device from address (hex) 4000 to 7FFF,  use it in a TDS2020DV piggy-back board on the TDS2020F card.In a H8/532 microprocessor: bottom of device from address (hex) 0000 to 3FFFFor details of changes since previous versions see file #UPDATES.TDS